# D-Flow FM – C-SUMO_precice – C-SUMO

# Introduction

We’re designing the communication between D-Flow FM and the C-SUMO_preCICE wrapper. We intent to keep the interface as clean/generic as possible and are thinking about moving code (desa-routine) from D-Flow FM to C-SUMO_preCICE.

# QandA Wilbert

- Q: Must the DIMR coupling still work when the preCICE coupling is added?
We are thinking about doing some refactoring that might break the DIMR coupling.
A: If the preCICE coupling can be used, then the DIMR coupling is not needed anymore.

- Q: Assumption: all points produced by C-SUMO are horizontally covered by the FM grid (when combining all partitions)
if not: it will be excluded, but it will still count in the weighting process
Is this a correct assumption?
A: The user is responsible for ensuring that this does not happen. It might be useful to stop with an error message if it does happen, but Wilbert will come back on this.

- Q: Above waterlevel or below bedlevel: handled by FM (no changes). 
Are changes needed?
A: Sources/sinks should NOT be neglected when they are completely below bedlevel or completely above waterlevel. So please add:
IF z_max < bedlevel THEN errormessage_and_stop
IF z_min > waterlevel THEN errormessage_and_stop

- Q: How important is "check_mixed_source_sink"?
The refactoring we have in mind will disconnect the generation of all sources and sinks from the FM-grid-awareness. 
Is this mix check still needed?
A: The user is responsible for ensuring that this does not happen. It might be useful to generate some message, but Wilbert will come back on this.

- Q: Discharge and const_excess: why is the base value averaged over intake locations of all diffusors?
A: That is wrong: the base value to use in case of const_excess is just the intake location of the current diffusor only.

- Q: How does “changing config during computation” work?
Who changes the config: C-SUMO or the user?
A: C-SUMO might do that in the future, for example in the following scenario:
During a computation, C-SUMO finds out that the locations of the ambient points are not optimal and changes them in the config file for the next computation.

- Q: Why is density re-computed using temperature and salinity, while array “rho” is already available?
A: That is not needed: just use rho as computed by D-Flow FM.

- Q: What is C-SUMO doing with the mdu-file?
Why is the mdu-file specified in the FF2NF file?
A:

- Q: Is it allowed that C-SUMO_preCICE uses/writes for the constituent names: Temperature, Salinity, A, B, C, ...?
Passing through the actual names of all constituents works a bit awkward with preCICE.
A:

- Q: How important is it that C-SUMO uses the exact z-locations from D-Flow FM? Currently, quantities are defined at the exact vertical locations of the D-Flow FM grid so no vertical interpolation is needed. The Proposal is that the C-SUMO config file contains an integer specifying the number of equidistant z-locations for all points of interest. The advantage is that preCICE can do the 3D interpolation from the D-Flow FM grid to these xyz-locations.
A:

- Q: Where is the DESA routine located?
The source code suggests that it’s inside Delft3D4-FLOW/D-Flow FM, while Roland’s PPT (22 April 2020) suggests it’s inside C-SUMO.
A:

# QandA Arthur

- Q: Are coupled source-sink locations handled correctly in D-Flow FM, also when the source is in a different partition than the sink? 
A: Yes

- Q: Are multiple sources/sinks in one D-Flow FM-3D-cell allowed? 
A: Technically: allowed, sinks: not safe-guarded

- Q: 3000+ sources/sinks, multiple per 3Dcell: lumping advised/necessary? 
A: It should not be a problem, can be done later if needed

- Q: Might conflicts pop up related to bubble screen implementation? 
A: No conflicts expected; bubble screens are added as “normal”, “static” source/sinks and will not affect the nearfield implementation in D-Flow FM

- Q: Avoid multiple re-allocating of source arrays. 
A: Possible to use some maximum dimension and keep everything allocated?

# QandA SEA-team

- Q: How to limit the constituent quantities
A: The number of quantities must be specified in the C-SUMO config file

- Q: How to define/exchange constituent names
A: In C-SUMO_preCICE, use the following standard names for the constituents:

- Q: Handle below bedlevel, above waterlevel correctly

# Remarks

- Key point is that we aim on refactoring the communication from C-SUMO_precice to D-Flow FM. The goal is to make the data being transferred independent of the D-Flow FM grid (mapping to the grid should happen inside D-Flow FM) and independent of the diffusor geometry (data entering D-Flow FM should just be a cloud of source/sink points). As a result, the routines related to DESA should be moved from D-Flow FM to C-SUMO_precice.

- PreCICE can only transfer real-type data on “a” grid

- Handle below bedlevel, above waterlevel correctly: handle on D-Flow FM side, for now: stop with an error message when a source/sink is:

  - Vertically outside the domain (above waterlevel or below bedlevel)

  - Horizontally outside all partitioned domains (use mpi reduce to detect)

- How to interpolate FM data to the C-SUMO “grid”: preCICE interpolation 3D